Knowledge Watchkeeping – Open Book Marking Sheet

| Trainer / Assessor Instructions for marking of assessment | - This answers within this marking sheet are comprehensive and technically correct as acceptable responses - Listed below are the questions asked and answers in red - Assessors must refer to these when considering the responses provided by participants - A course participant is not expected to provide answers to that level of detail, instead for a satisfactory response, the participant’s answers should be within the range of model answers and meet the minimum required number of responses provided and indicate a basic level of knowledge and understanding of the topic - Each question must be completed correctly, and the assessor must indicate whether the response is satisfactory in the column against each question on the assessment - If a verbal answer is given for any questions, the assessor is to document the participant’s answers and initial next to it |
| --- | --- |
| Reasonable Adjustment | The purpose of reasonable adjustment is to make it possible for learners to participate fully. A reasonable adjustment in teaching, learning and assessment activity needs to be justifiable and uphold the integrity of the qualification. Inherent requirements are the fundamental parts of a course that must be met by all students for them to be deemed competent. They are the abilities, skills and knowledge students need to complete the course — those components which, if removed, would compromise the learning outcomes - Assessors may accept variations of an acceptable response in marking the assessment for example where the benchmark is ‘adequate lighting’ the assessor may also choose to accept variations such as ‘enough lighting, sufficient lighting, good bright lighting.’ - Assessors must ensure the answers provided are comparable to the meaning and intent of the acceptable responses provided. |

Watchkeeping

##### How do you determine if a risk of collision exists with another vessel. Range decreasing Bearing remain steady

##### You are the Coxswain of a power-driven vessel making way. The following lights are seen ahead on a steady bearing. Identify the type of vessel and your actions required by the Collision Regulations ##### I003-PC1.1, 3.1, 3.2, P5, K8, K11

##### Type of vessel VESSEL ENGAGED IN FISHING OTHER THAN TRAWLING ##### Are you the stand-on or give-way vessel_ GIVE WAY VESSEL ##### Course of action: SLOW DOWN or STOP TO ALLOW VESSEL PASSES AHEAD. OBSERVE ACTION OF OTHER VESSEL. 360 TURN HEADING TO STARBOARD. ##### TURN TO PORT AND PASS ASTERN OF VESSEL.

##### Type of vessel_ POWER DRIVEN VESSEL <50m, SEEN FROM AHEAD ##### Are you the stand-on or give-way vessel Give-way ##### Course of action: TURN TO STARBOARD

##### Type of vessel_ PDV < 50m ENGAGED IN TOWING (<200m) WITH VESSEL BEING TOWED ##### Are you the stand-on or give-way vessel GIVE WAY Course of action: TURN TO STARBOARD

##### List two (2) types of vessels this light could be: - VESSEL AT ANCHOR <100m - <7m, less than 7 knots - Stern Light - Something being towed

##### Complete the blanks in the tables below Vessel lights; Vessel type and Operation; Day Shapes. VESSEL CONSTRAINED BY DRAFT. VESSEL NOT UNDER COMMAND. Fishing vessel seen from astern. Vessel Towing >200m. Sound signals (fill in the blank); Vessel Type and Operation (fill in the blank). Rapid ringing of the bell for 5 seconds every minute; Vessel at anchor <100m. In Restricted Visibility 1 Prolonged + 2 Short Every 2 minutes; Sailing, Fishing, NUC, RAM, CBD, Towing Underway.

##### In restricted visibility you hear a fog signal consisting of one (1) prolonged blast at intervals of about 2 minutes. What vessel is this. (Circle your answer) ##### A power-driven vessel underway, but stopped and making no way through the water ##### A power-driven vessel making way through the water ##### A vessel being towed by another vessel ##### A vessel NUC

##### A power-driven vessel in sight of sailing a vessel makes a sound signal consisting of (2) two short blasts. What does this mean. (Circle your answer) ##### I am altering my course to port ##### I am altering my course to starboard ##### I am operating astern propulsion ##### Your intentions are not understood

- What type of mark is this ___South Cardinal_________________________ In which direction is Safe Water. ________South___________________________ What colour is the light. _____White_________________ What is the light flashing sequence ____Q or VQ, six short + 1 Long_____________

- What type of mark is this. Port lateral mark What is the light colour and Rhythm. Red, any rythmn except Gp (2 + 1)