SMT

Coxswain Deck

Knowledge Assessment Watchkeeping

(Closed book)

Marking Sheet

Note for assessors

The answers in this marking sheet are comprehensive and technically correct.

A course participant is not expected to provide answers to that level of detail. Instead, for a satisfactory response, the participant’s answers should be within the range of model answers provided and indicate a basic level of knowledge and understanding of the topic.

Watchkeeping

- When two (2) vessels are approaching one another, a risk of collision can exist when. - The distance of approaching vessel is closing. - The compass bearing of approaching vessel is steady. - An appreciable bearing change is evident when approaching a large, close vessel. - All of the above

Two (2) power-driven vessels are crossing and there is a risk of collision. Draw the actions for each vessel to avoid collision. B A Who is the stand-on vessel. Vessel B What is the action of the give-way vessel. Make bold and early turn to starboard-Pass behind vessel ‘B’

- Vessel ‘A’ is overtaking Vessel ‘B’. Draw which way Vessel ‘A’ can overtake Vessel ‘B’. Vessel ‘A’ can overtake Vessel “B”. - Portside only - Starboard side only. - Either side (Pending safe navigation) What action shall Vessel ‘B’ take whilst being overtaken. Maintain course and speed, observe other vessel to ensure safe navigation

- In a distress situation, list THREE ways you would raise the attention of another vessel within sight. Range of answers can include: pyrotechnics – rocket (parachute) flares, smoke signal; slowly raising and lowering arms; square flag with ball above or below; code flag N over C; sound signals – continuous sounding of fog horn or signalling device, explosive signal at intervals of 1 minute; radio, smouldering bucket Answer should not include EPIRB – this does not attract attention of other vessel

- On which side of this Cardinal Mark is safe water. North East South West

- Describe the light and flashing characteristic of an Isolated Danger Mark Colour White Flashing Flashing (2)

- When leaving Harbour, which side of your vessel should you keep this mark. ## Port Starboard

- Using the chart provided, draw a safe route from Position A to Position B.

- When determining a safe speed for your vessel, list 5 factors the Colregs require you to consider. (i) The state of visibility; (ii) the traffic density including concentrations of fishing vessels or any other vessels; (iii) the manoeuvrability of the vessel with special reference to stopping distance and turning ability in the prevailing conditions; (iv) at night the presence of background light such as from shore lights or from back scatter of her own lights; (v) the state of wind, sea and current, and the proximity of navigational hazards; (vi) the draught in relation to the available depth of water.