Knowledge Seamanship

Marking Sheet

| Trainer / Assessor Instructions for marking of assessment | - This answers within this marking sheet are comprehensive and technically correct as acceptable responses - Listed below are the questions asked and answers in red - Assessors must refer to these when considering the responses provided by participants - A course participant is not expected to provide answers to that level of detail, instead for a satisfactory response, the participant’s answers should be within the range of model answers and meet the minimum required number of responses provided and indicate a basic level of knowledge and understanding of the topic - Each question must be completed correctly, and the assessor must indicate whether the response is satisfactory in the column against each question on the assessment - If a verbal answer is given for any questions, the assessor is to document the participant’s answers and initial next to it |
| --- | --- |
| Reasonable Adjustment | The purpose of reasonable adjustment is to make it possible for learners to participate fully. A reasonable adjustment in teaching, learning and assessment activity needs to be justifiable and uphold the integrity of the qualification. Inherent requirements are the fundamental parts of a course that must be met by all students for them to be deemed competent. They are the abilities, skills and knowledge students need to complete the course — those components which, if removed, would compromise the learning outcomes - Assessors may accept variations of an acceptable response in marking the assessment for example where the benchmark is ‘adequate lighting’ the assessor may also choose to accept variations such as ‘enough lighting, sufficient lighting, good bright lighting.’ - Assessors must ensure the answers provided are comparable to the meaning and intent of the acceptable responses provided. |

| Part A – Emergency preparedness |  |
| --- | --- |
| - To improve the safety of your vessel and crew prior to crossing a bar, list TWO actions you could take for each of the following. Crew. All crew up in safe area-Not below deck Monitor wave sets and safe water potential Crew briefed on crossing Tide and Weather PPE fitted and checked Roles assigned to crew (lookout, bilge, etc) Vessel stability All equipment that has to be left on deck is made secure. (lower heavy items) Watertight doors, hatch covers and openings to below deck are secured and watertight. Freeing ports (scuppers) are free and working and a deck life line rigged. Bilges are pumped dry and tanks are either completely full or empty. Regulate the speed of the boat so that it is positioned behind the crest of the wave. Engineering checks. Full range of propulsion and steering Sounder and ENA checked and working correctly Radio check Fuel levels Trim vessel Bilge pumping system |  |
| - a) List two (2) signs of approaching bad weather. - Increase in wave height from storm direction - Rain increasing - Wind increasing - Cloud formation -Cirrus - Dropping temperature - Dropping barometric pressure - Radio forecast observations - Forecasted updated received - What are the dangers of an approaching thunder storm. Adverse weather-High winds, waves increasing, lighting strikes, hail damage - Match the following with their average wind strengths. Strong wing warning 48-63 knots Gale warning 26-33 knots Hurricane force wind warning 34-47 knots Storm force wind warning 64 or higher knots |  |
| - Tropical Revolving Storms (TRS) impacted which areas of Australia. North of the line of the tropic of Capricorn. Northern Australia What time of the year are these more likely to occur. ### November to April |  |
| - List six (6) courses of action you would take following a grounding of your vessel. Raise the alarm Stop engine Display day shape or lights Determine position on chart Tide check Inspection of hull integrity Deploy anchor to support re-floating Remove weight on vessel-Water if possible Inform port authority / other vessels Call muster Head count |  |
| - List FIVE actions to be taken if a person falls overboard from your vessel. Throw a life buoy or any other flotation device as close to the person as possible. If unable to immediately locate the person overboard send an urgency message (PAN, PAN). Yell out ‘man overboard’ and the side to which he/she fell. Watchkeeper hits man overboard button on the GPS/plotter. Turn the vessel towards the side the person went overboard so as to turn the propeller away from them. Make sure visual contact is maintained at all times and someone points to the person in the water. Notify the crew of the pickup side of vessel. Stop the vessel when the person is near enough to be retrieved. Ensure propeller is not operating with the person alongside. |  |
| - You hear a distress (Mayday) call on the VHF radio, list THREE actions you would take. monitor the distress situation for as long as possible Wait for governing body to respond make an entry into the vessel’s record (log) book Determine location in relation to you Calculate fuel required to safely support Offer assistance if safe to do so. |  |
| - A risk assessment identifies the following three (3) high risk FIRE areas. List two (2) controls for each that can be implement to reduce the risk of a fire. Risk area; Control/s. Refuelling points; Fire watch, No smoking, No hot works Shutdown electrical systems, Shutdown engines.. Galley; Keep clean, safe stowage of oils and other flammable items, ensure ventilation, islote gas when not in use, regular cleaning of extraction fans. Engine room; Remove all rags, bilges clean, repair oil leaks, repair fuel leaks. |
| - List three considerations you would make before setting up to tow another vessel - Weather - Distance to support and return to safety i.e. Fuel - Safety of crew - Communications - Power to weight ratio of your vessel against that of the vessel requesting a tow - Suitable equipment to undertake tow - Insurance obligations - Operational areas and survey restrictions |  |
| PART B – Vessel Handling and watertight integrity |  |
| - If travelling in shallow water, list two ways to minimise the risk of your vessel squatting. Reduce speed, find deeper water |  |
| - List two (2) manoeuvring characteristics of the following single engine propulsion systems: Fixed propeller, Inboard shaft drive Slow steering response no trim or tilt capability from engine Transverse thrust Limited steerage when going astern Outboard with tiller steering: cavitation Quicker response at helm Trim feature Manoeuvre tiller in the opposite direction to you want to go Jet Unit: Opposite reaction from helm to conventional steering when going astern Able to hold specific position Precise manoeuvring Responsive manoeuvring Manoeuver in all direction |  |
| - How can the risk of capsizing or swamping be reduced in large seas? Bow into the seas, stow heavy items low, cargo securely lashed, remove free surface effect, clear scuppers/ freeing ports, close watertight doors and opening, avoid autpilot |  |
| - List two (2) ways you can reduce interaction (coming too close) when approaching a large vessel or structure. Manual Helm No Auto Pilot Reduce speed Make contact with other vessel Plan approach with crew |  |
| - Fill in the names of each of the mooring lines below: 1__Stern_________ 2_Forward Springer___ 3___Aft springer_____ 4__Bow line______ |  |
| ### 5. In your own words describe the following stability terms Heel: Heeling is the boat “tipping” to one side or the other; it is caused primarily by the force of the wind, or other external factors List: A list is a "permanent" inclination of a ship, caused by one of the following conditions 1. Off-Center Weight (99% of the time) 2. Negative GM (1% of the time) 3. Combination of Off-Center Weight and -GM Trim: The trim of a ship describes its floating position in length direction, namely if the bow or the aft of the vessel is deeper submerged into the water. |  |
| Part C – Vessel systems |  |
| - List THREE safety precautions you must consider when refuelling a vessel. - Check port regulation. Secure the boat firmly to the wharf. - Block up all freeing ports (scuppers) so that any spillage stays on board. - Have firefighting equipment nearby. - Spill kits at the ready - Make sure the fuelling hose is correctly earthed. - Have an approximate estimate of how much fuel you need. - Turn off any LPG systems. - Shut down all unnecessary machinery and engines. - No smoking, welding or other activities that have naked flames or cause sparks. - Make sure you are in constant communication with the fuelling person. |  |
| - List THREE probable reasons for a engine to overheat? Obstruction of the cooling water intakes and outlets, Water circulation pump failure, Overloading the engine, Incorrect oil/fuel mixture, overloaded, incorrect propeller |  |
| - List FOUR possible reasons an outboard engine may fail to start Fuel System: 1. No fuel or not primed 2. Disconnected line 3. Line fitted backward 4. Blocked filters Electrical System - No power in batteries - Kill switch off - In gear - What action do you take if the equipment if unsafe? Lock Out Tag Out and report to vessel owner/supervisor/maintenance team/log deficiency |  |